Scott Dann

MINIMUM EQUIPMENT LIST

Mitsubishi MU-2B

N82WC	s/n- 393SA

Revision: 11

Dated: 02/06/2009

Scott Dann	Page:	I

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Table of Contents

SYSTEM	SYSTEM        			PAGE NO.

--		Table of Contents                     		I

--		Log of Revision				II

--		Control Page					III, IV

--		Highlights of Change				V, VI, VII, VIII

--		Definitions					IX thru XIV

--		Preamble					XV, XVI

--		Guidelines for (O) & (M) Procedures		XVII thru XXI

21		Air Conditioning				21-1

22		Auto Flight					22-1, 2

23		Communications				23-1 thru 4

24		Electrical Power				24-1

25		Equipment/Furnishings			25-1 thru 5

26		Fire Protection					26-1

27		Flight Controls					27-1

28		Fuel						28-1

30		Ice and Rain Protection			30-1, 2

31		Indicating/Recording Systems		31-1, 2

32		Landing Gear					32-1

33		Lights						33-1, 2

34		Navigation					34-1 thru 15

35		Oxygen					35-1

46		Information Systems				46-1

52		Doors						52-1

61		Propellers					61-1

76		Engine Controls				76-1

Scott Dann	Page:	II

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Log of Revisions

REV. NO.	DATE                PAGE NUMBERS						INITIALS

Original		03/15/1979	Original

1		06/13/1979	All pages

2		03/30/1982	All Pages

3		06/11/1985	All pages

4		07/24/1985	Page 33-1 (Pen & Ink - Item 4)

5		04/14/1987	Page 76-1

6		05/16/1989	All Pages

7		06/26/1989	Highlights of Rev., Definitions

7		06/26/1989	Preamble

8		06/19/1990	Highlights of Rev., Definitions

8		06/19/1990	Preamble, Guidelines

8		06/19/1980	23-1, 24-1, 25-1, 25-2, 30-2

8		06/19/1990	33-1, 34-2, 52-1

8a		04/15/1991	Highlights of Rev.

8a		04/15/1991	23-1

9		03/25/1994	Highlights of Rev., Definitions

9		03/25/1994	21-1, 22-1, 23-1, 24-1, 25-1

9		03/25/1994	25-2, 26-1, 28-1, 30-1, 30-2

9		03/25/1994	31-1, 33-1, 33-2, 34-1, 34-2

9		03/25/1994	34-3, 35-1, 52-1, 61-1, 76-1

9a		12/14/1995	Highlights of Rev., Definitions

9a		12/14/1995	Guidelines

9a		12/14/1995	28-1

10		06/03/2003	Highlights of Rev., Definitions

10		06/03/2003	Guidelines

10		06/03/2003	21-1, 22-1, 23-1, 23-2, 25-1

10		06/03/2003	25-2, 25-3, 26-2, 28-2, 30-2

10		06/03/2003	31-1, 33-1, 33-2, 34-1, 34-2

10		06/03/2003	34-3, 34-4, 34-5, 34-6, 34-7

10		06/03/2003	34-8, 34-9, 34-10, 34-11

11		02/06/2009	Highlights of Change, Definitions

11		02/06/2009	Preamble, Guidelines (O&M)

11		02/06/2009	22-1, 22-2, 23-1, 23-2, 23-3, 23-4

11		02/06/2009	25-1, 25-2, 25-3, 25-4, 25-5, 27-1

11		02/06/2009	28-1, 30-1, 30-2, 31-1, 31-2, 32-1

11		02/06/2009	33-1, 33-2, 34-1, 34-2, 34-3, 34-4

11		02/06/2009	34-5, 34-6, 34-7, 34-8, 34-9, 34-10

11		02/06/2009	34-11, 34-12, 34-13, 34-15, 35-1

11		02/06/2009	46-1, 52-1

Scott Dann											Page:	III

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Control Page

SYSTEM					PAGE		REV NO.	CURRENT DATE

Cover Page			                               -       	11      		02/06/2009

Table of Contents			                  I       	11      		02/06/2009

Log of Revisions			                  II  		11      		02/06/2009

Control Page 				                  III 		11      		02/06/2009

IV  		11      		02/06/2009

Highlights of Change   			                  V  		11      		02/06/2009

VI  		11      		02/06/2009

VII 		11      		02/06/2009

VIII   	11      		02/06/2009

Definitions		                                            IX 		11      		02/06/2009

X  		11      		02/06/2009

XI  		11      		02/06/2009

XII 		11      		02/06/2009

XIII  	 	11      		02/06/2009

XIV  		11      		02/06/2009

Preamble                          		                  XV   	 	11      		02/06/2009

XVI    	11      		02/06/2009

Guidelines for (O) & (M) Procedures   		     XVII   	11      		02/06/2009

XVIII   	11      		02/06/2009

XIX     	11      		02/06/2009

XX  		11      		02/06/2009

XXI  		11      		02/06/2009

21         			                               21-1 	10       		06/03/2003

22 				                               22-1    	11      		02/06/2009

22-2    	11      		02/06/2009

23 				                               23-1    	11      		02/06/2009

23-2    	11      		02/06/2009

23-3    	11      		02/06/2009

23-4    	11      		02/06/2009

24 	                 			                  24-1   	 9  		03/25/1994

25 				                               25-1    	11      		02/06/2009

25-2    	11      		02/06/2009

25-3    	11      		02/06/2009

25-4    	11      		02/06/2009

25-5    	11      		02/06/2009

26   				                               26-1   	10      		06/03/2003

27 				                               27-1    	11      		02/06/2009

28  				          	                  28-1    	11      		02/06/2009

30                                       			     30-1 	11      		02/06/2009

30-2    	11      		02/06/2009

31                                       			     31-1    	11      		02/06/2009

31-2    	11      		02/06/2009

32                                       			     32-1    	11      		02/06/2009

33                                       			     33-1    	11      		02/06/2009

33-2    	11      		02/06/2009

34                                       			     34-1    	11      		02/06/2009

34-2    	11      		02/06/2009

34-3    	11      		02/06/2009

34-4    	11      		02/06/2009

Scott Dann	Page:	IV

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Control Page

SYSTEM					PAGE		REV NO.	CURRENT DATE

34-5    		11      		02/06/2009

34-6    		11      		02/06/2009

34-7    		11      		02/06/2009

34-8    		11      		02/06/2009

34-9    		11      		02/06/2009

34-10   		11      		02/06/2009

34-11   		11      		02/06/2009

34-12   		11      		02/06/2009

34-13   		11      		02/06/2009

34-14   		11      		02/06/2009

34-15   		11      		02/06/2009

35	                                       		35-1    		11      		02/06/2009

46	                                       		46-1    		11      		02/06/2009

52                                       			52-1    		11      		02/06/2009

61                                       			61-1    		9        		03/25/1994

76                                       			76-1    		9        		03/25/1994

Scott Dann	Page:	V

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Highlights of Change

Cover Page		Updated to Revision 11 and incorporate MKC AEG Cover Page format

Table of Contents	Updated to incorporate Revision 11 changes

Log of Revisions	Updated to incorporate Revision 11 changes

Control Pages		Updated to incorporate Revision 11 changes

Highlights of Change	Updated to incorporate Revision 11 changes

Preamble		Updated to reference current Policy Letter 34 (or 36 for Part 91-only operations) revision to

include in operator’s MEL

Definitions		Updated to reference current Policy Letter 25 revision for applicable portions to include in

operator’s MEL

Guidelines (O & M)	Updated to incorporate Revision 11 changes

ATA 22-1  		Update autopilot relief to comply with SFAR and add individual mode relief

ATA 22-2 	 	Update Autopilot Disconnect/trim Interrupt Switch relief to address Trim Interrupt functions

ATA 22-3  		Add relief for Yaw Damper System

ATA 22-4  		Include relief for Flight Directors from Chapter 34 and add specific systems

ATA 23-2  		Correct number installed and Remarks for Cockpit Speaker System

ATA 23-3  		Change Item Description to Audio Panel/Amplifier because the unit is integrated, correct Number 				Installed and Remarks

ATA 23-4 	 	Cockpit Voice Recorder relief updated IAW PL-29

ATA 23-5	  	Boom Microphone relief updated IAW PL-58

ATA 23-6	  	HF relief updated IAW PL-106

ATA 23-7	  	Passenger Address System relief added IAW PL-09

ATA 23-8  		Handheld Microphone relief added

ATA 23-9	 	Static Discharge Wick relief added

ATA 25-2		Passenger Seat relief updated IAW PL-79

ATA 25-2-2		Passenger Seat Armrest relief added

ATA 25-3		Change Item to Flotation/Overwater Equipment and list specific equipment

Scott Dann	Page:	VI

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Highlights of Change

ATA 25-4		ELT relief updated IAW PL-120 and add relief for Remote ELT Switch

ATA 25-5		Emergency Medical Equipment relief expanded and updated IAW PL-73

ATA 25-6		Passenger Convenience Items relief replaced by NEF Items relief IAW PL-116

ATA 25-7		Add “Fasten Seat Belt” placards to Item Description

ATA 25-8	Cabin Storage relief updated IAW PL-104

ATA 25-10		Change Item Description for waste Receptacle Doors and add ***

ATA 25-11		Add relief for EMS Equipment

ATA 27		  	Add Chapter 27 FLIGHT CONTROLS

ATA 27-1		Electric Pitch Trim relief added

ATA 28-1	Correct Auto Fuel Transfer Remarks and add second relief to preclude the need to defuel Outer Tanks provided fuel is within balance limits

ATA 30-4		Correct applicability of anti ice load meter and provide separate relief for each applicability

ATA 30-10 		Pitot Heaters Remarks corrected to separate relief format per PL-31

ATA 30-11		Ice Detector System relief added as optional item

ATA 31-3  	Flight Data Recorder relief updated IAW PL-87, Removed *** because FDR may be required with 10 or more passengers

ATA 32			Add Chapter 32 LANDING GEAR

ATA 32-1		Parking Brake relief added IAW PL-92

ATA 33-1-1		Tail Position Light relief added IAW PL-91

ATA 33-5		Wing Illumination Light relief updated to current Wing Ice Light relief policy and AD 96-25-02

ATA 33-9		Cabin Interior Lights relief updated to current policy

ATA 33-10		Baggage Compartment Light System relief added

ATA 34-1 		Correct Altimeter relief Remarks format and address EFIS

ATA 34-2 		Correct Airspeed relief Remarks format and address EFIS

ATA 34-3 		Correct Attitude Indicator relief Remarks format and address EFIS

Scott Dann	Page:	VII

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Highlights of Change

ATA 34-4 		Correct Heading Indicator relief Remarks format and address EFIS

ATA 34-5 	Correct Rate-of-Turn Indicator relief Remarks format, Separate relief to address Yaw Damper affected by Rate-of-Turn Indicator

ATA 34-6		Correct Vertical Speed Indicator relief Remarks format

ATA 34-7		Delete Flight Director from Chapter 34 and expand relief in Chapter 22

ATA 34-8 		Radar Altimeter remarks added to address operational procedures

ATA 34-10		Transponder relief updated IAW PL-76, Add relief for Ident buttons on yoke

ATA 34-13		Change Item Description to reflect available Navigation Equipment

ATA 34-15	 	Altitude Alerting System relief updated to incorporate RVSM NOTE

ATA 34-17 		Add NOTE about ADF to RMI relief

ATA 34-19 		TCAS I relief updated IAW PL-32

ATA 34-20 		TCAS II relief updated IAW PL-32

ATA 34-21 		GPWS relief updated and incorporated into TAWS relief IAW PL-54

ATA 34-22 		Non-Stabilized Magnetic Compass relief updated IAW PL-10, Add AHRS

ATA 34-23 		Windshear Warning and Flight Guidance System updated IAW PL-67

ATA 34-24 		Windshear Detection and Avoidance System updated IAW PL-67

ATA 34-25 		Format Item for Flight Management System Navigation Database

ATA 34-26		Format Item for Navigation Management Systems Navigation Database

ATA 34-28		Standby Attitude Indicator relief updated IAW PL-111

ATA 35-1		Delete Oxygen System relief because only one system and pilot must have oxygen available for

Emergency Procedures

ATA 35-1-1		Add relief for Individual Passenger Oxygen Masks and Add NOTE for operative pilot oxygen masks

always required

ATA 35-2		Protective Breathing Equipment relief added IAW PL-43

ATA 46			Add Chapter 46 INFORMATION SYSTEMS

ATA 46-1		Electronic Flight Bag relief added IAW PL-121

ATA 52-1		Add applicable indicators to Door Warning System Item description.  Remove Remark to operate

unpressurized

Scott Dann	Page:	VIII

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Highlights of Change

ATA 46-1		Electronic Flight Bag relief added IAW PL-121

ATA 52-1		Add applicable indicators to Door Warning System Item description.  Remove Remark to operate

unpressurized

Scott Dann	Page:	IX

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Definitions

1.   System Definitions.

System numbers are based on the Air Transport Association (ATA) Specification Number 100 and items are numbered sequentially.

a. “Item" (Column 1) means the equipment, system, component, or function listed in the "Item" column.  Repair interval categories (A, B, C and D) are listed on right side of column 1.  Repair intervals are described in Definition 22.

b. "Number Installed" (Column 2) is the number (quantity) of items normally installed in the aircraft.  This number represents the aircraft configuration considered in developing this MMEL.  Should the number be a variable (e.g., passenger cabin items) a number is not required.

c. "Number Required for Dispatch" (Column 3) is the minimum number (quantity) of items required for operation provided the conditions specified in Column 4 are met.

NOTE:  Where the MMEL shows a variable number required for dispatch, the MEL must reflect the actual number required for

dispatch or an alternate means of configuration control approved by the Administrator.

d. "Remarks or Exceptions" (Column 4) in this column includes a statement either prohibiting or permitting operation with a specific number of items inoperative, provisos (conditions and limitations) for such operation, and appropriate notes.

e. A vertical bar (change bar) in the margin indicates a change, addition or deletion in the adjacent text for the current revision of that page only.  The change bar is dropped at the next revision of that page.

Scott Dann	Page:	IX

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Definitions

2.  "Airplane/Rotorcraft Flight Manual" (AFM/RFM) is the document required for type certification and approved by the responsible FAA Aircraft Certification Office. The FAA approved AFM/RFM for the specific aircraft is listed on the applicable Type Certificate Data Sheet.

3.  "As required by FAR" means that the listed item is subject to certain provisions (restrictive or permissive) expressed in the

Federal Aviation Regulations operating rules.  The number of items required by the FAR must be operative.  When the listed item is not required by FAR it may be inoperative for time specified by repair category.  The term “14 CFR” may be substituted for “FAR” in MMEL’s or MEL’s.

4.  Each inoperative item must be placarded to inform and remind the crewmembers and maintenance personnel of the equipment condition.

NOTE:  To the extent practical, placards should be located adjacent to the control or indicator for the item affected; however, unless otherwise specified, placard wording and location will be determined by the operator.

5.  "-" symbol in Column 2 and/or Column 3 indicates a variable number (quantity) of the item installed.

6.  "Deleted" in the remarks column after a sequence item indicates that the item was previously listed but is now required

to be operative if installed in the aircraft.

7.  As used in MMEL’s, "ER" refers to Extended Operations (ETOPS) of an airplane with operational approval to conduct ETOPS in accordance with the applicable regulations

8.  "Federal Aviation Regulations" (FAR) means the applicable portions of the Federal Aviation Act and Federal Aviation

Regulations.

9.  "Flight Day" means a 24 hour period (from midnight to midnight) either Universal Coordinated Time (UCT) or local time,

as established by the operator, during which at least one flight is initiated for the affected aircraft.

10.  "Icing Conditions" means an atmospheric environment that may cause ice to form on the aircraft or in the engine(s).

11.  Alphabetical symbol in Column 4 indicates a proviso (condition or limitation) that must be complied with for operation with the listed item inoperative.

Scott Dann	Page:	X

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Definitions

12.  "Inoperative" means a system and/or component malfunction to the extent that it does not accomplish its intended purpose and/or is not consistently functioning normally within its approved operating limit(s) or tolerance(s).

13.  "Notes:" in Column 4 provides additional information for crewmember or maintenance consideration.  Notes are used to identify applicable material which is intended to assist with compliance, but do not relieve the operator of the responsibility for compliance with all applicable requirements.  Notes are not a part of the provisos.

14.  Inoperative components of an inoperative system:

Inoperative items which are components of a system which is inoperative are usually considered components directly associated with and having no other function than to support that system.  (Warning/caution systems associated with the inoperative system must be operative unless relief is specifically authorized per the MMEL).

15.  "(M)" symbol indicates a requirement for a specific maintenance procedure which must be accomplished prior to operation with the listed item inoperative.  Normally these procedures are accomplished by maintenance personnel; however, other personnel may be qualified and authorized to perform certain functions.  Procedures requiring specialized knowledge or skill, or requiring the use of tools or test equipment should be accomplished by maintenance personnel.  The satisfactory accomplishment of all maintenance procedures, regardless of who performs them, is the responsibility of the operator.

Appropriate procedures are required to be published as part of the operator's manual or MEL.

16.  "(O)" symbol indicates a requirement for a specific operations procedure which must be accomplished in planning for and/or operating with the listed item inoperative.  Normally these procedures are accomplished by the flight crew; however, other personnel may be qualified and authorized to perform certain functions.  The satisfactory accomplishment of all procedures, regardless of who performs them, is the responsibility of the operator.  Appropriate procedures are required to be published as a part of the operator's manual or MEL.

NOTE:  The (M) and (O) symbols are required in the operator's MEL unless otherwise authorized by the Administrator.

Scott Dann	Page:	XI

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Definitions

17.  "Deactivated" and "Secured" means that the specified component must be put into an acceptable condition for safe flight.  An acceptable method of securing or deactivating will be established by the operator.

18.  "Visual Flight Rules" (VFR) is as defined in FAR Part 91. This precludes a pilot from filing an Instrument Flight Rules (IFR) flight plan.

19.  "Visual Meteorological Conditions" (VMC) means the atmospheric environment is such that would allow a flight to proceed under the visual flight rules applicable to the flight.  This does not preclude operating under Instrument Flight Rules.

20.  "Visible Moisture" means an atmospheric environment containing water in any form that can be seen in natural or artificial light; for example, clouds, fog, rain, sleet, hail, or snow.

21.  "Passenger Convenience Items" means those items related to passenger convenience, comfort or entertainment such as, but not limited to, galley equipment, movie equipment, ash trays, stereo equipment, overhead reading lamps, etc.

22.  Repair Intervals:  All users of an MEL approved under FAR 121, 125, 129 and 135 must effect repairs of inoperative systems or components, deferred in accordance with the MEL, at or prior to the repair times established by the following letter designators:

Category A.  Items in this category shall be repaired within the time interval specified in the remarks column of the operator's approved MEL.  For time intervals specified in “flight days”, the day the malfunction was recorded in the aircraft maintenance record/logbook is excluded.  For all other time intervals (flights, flight legs, cycles, hours, etc.) repair tracking begins at the point when the malfunction is deferred in accordance with the operator’s approved MEL.

Category B.  Items in this category shall be repaired within three (3) consecutive calendar days (72 hours), excluding the day the malfunction was recorded in the aircraft maintenance record/logbook.  For example, if it were recorded at 10 a.m. on January 26th, the three day interval would begin at midnight the 26th and end at midnight the 29th.

Category C.  Items in this category shall be repaired within ten (10) consecutive calendar days (240 hours), excluding the day the malfunction was recorded in the aircraft maintenance record/logbook.  For example, if it were recorded at 10 a.m. on

January 26th, the 10 day interval would begin at midnight the 26th and end at midnight February 5th.

Category D.  Items in this category shall be repaired within one hundred and twenty (120) consecutive calendar days (2880

hours), excluding the day the malfunction was recorded in the aircraft maintenance log and/or record.

An operator who has the authorization to use an MMEL also has the authority to approve extensions to the maximum repair interval for Category B and C items provided the responsible Flight Standards District Office (FSDO) is notified within 24 hours of the MEL extension.  The operator is not authorized to extend A and D items in the MEL.  Misuse of the MEL extension authority may result in the operator’s OpSpecs/MSpecs being amended by removing the authority for the operator to use the MEL extension authority and/or use and MEL.

Scott Dann	Page:	XII

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Definitions

23. Electronic fault alerting system - General

New generation aircraft display system fault indications to the flight crew by use of computerized display systems.  Each        aircraft manufacturer has incorporated individual design philosophies in determining the data that would be represented.  The following are customized definitions (specific to each manufacturer) to help determine the level of messages affecting the aircraft's dispatch status.  When preparing the MEL document, operators are to select the proper Definition No. 23 for their aircraft, if appropriate.

NO CUSTOMIZED DEFINITIONS OF FAULT ALERTING ARE APPLICABLE TO THE MISTUBISHI MU-2B AIRCRAFT

24.  "Administrative control item" means an item listed by the operator in the MEL for tracking and informational purposes.  It

may be added to an operator's MEL by approval of the Principal Operations Inspector provided no relief is granted, or provided conditions and limitations are contained in an approved document (i.e. Structural Repair Manual, airworthiness directive, etc.).

If relief other than that granted by an approved document is sought for an administrative control item, a request must be        submitted to the Administrator.  If the request results in review and approval by the FOEB, the item becomes an MMEL item rather than an administrative control item.

25.  "***" symbol in Column 1 indicates an item which is not required by regulation but which may have been installed on some

models of aircraft covered by this MMEL.  This item may be included on the operator's MEL after the approving office has       determined that the item has been installed on one or more of the operator's aircraft.  The symbol, however, shall not be carried forward into the operator's MEL.  It should be noted that neither this policy nor the use of this symbol provide authority to install or remove an item from an aircraft.

Scott Dann	Page:	XIII

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Definitions

26.  "Excess Items" means those items that have been installed that are redundant to the requirements of the FARs.

27.  "Day of Discovery" is the calendar day an equipment/instrument malfunction was recorded in the aircraft.

28.  “Considered Inoperative”, as used in the provisos means that item must be treated for dispatch, taxi and flight purposes as though it were inoperative.  The item shall not be used or operated until the original deferred item is repaired.  Additional actions include: documenting the item on the dispatch release (if applicable), placarding and complying with all remarks, exceptions and related MMEL provisions, including any (M) and (O) procedures and observing the repair category.

29.  “Is not used” in the provisos, remarks or exceptions for an MMEL item may specify that another item relieved in the MMEL ‘Is not used”.  In such cases, crewmembers should not activate, actuate, or otherwise utilize that component or system under normal operations.  It is not necessary for the operators to accomplish the (M) procedures associated with the item.  However, operational requirements must be complied with, and an additional placard must be affixed, to the extent practical, adjacent to the control or indicator for the item that is not used to inform crewmembers that a component or system is not to be used under normal circumstances.

Scott Dann	Page:	XIV

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Definitions

30.  Nonessential equipment and furnishings (NEF) are those items installed on the aircraft as part of the original type certification, supplemental type certificate, or other form of alteration that have no effect on the safe operation of flight and would not be required by the applicable certification rules or operational rules.  They are those items that if inoperative, damaged or missing have no effect on the aircraft’s ability to be operated safely under all operational conditions.  These nonessential items may be installed in areas including, but not limited to, the passenger compartment, flight deck area, service areas, cargo areas, crew rest areas, lavatories and galley areas.  NEF items are not items already identified in the MEL or CDL of the applicable aircraft.  They do not include items that are functionally required to meet the certification rule or for compliance with any operational rule.  Operator’s NEF process shall not provide for deferral of items within serviceable limits identified in the manufacturer’s maintenance manual or operator’s approved maintenance program such as wear limits, fuel/hydraulic leak rates, oil consumption, etc.  Cosmetic items that are fully serviceable but worn or soiled may be deferred under an operator’s NEF process.

Scott Dann	Page:	XV

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Preamble

The following is applicable for authorized certificate holders operating under Federal Aviation Regulations (FAR) Parts 121, 125, 129, 135: The FAR require that all equipment installed on an aircraft in compliance with the Airworthiness Standards and the Operating Rules must be operative.  However, the Rules also permit the publication of a Minimum Equipment List (MEL) where compliance with certain equipment requirements is not necessary in the interests of safety under all operating conditions.  Experience has shown that with the various levels of redundancy designed into aircraft, operation of every system or installed component may not be necessary when the remaining operative equipment can provide an acceptable level of safety.  A Master Minimum Equipment List (MMEL) is developed by the FAA, with participation by the aviation industry, to improve aircraft utilization and thereby provide more convenient and economic air transportation for the public.  The FAA approved MMEL includes those items of equipment related to airworthiness and operating regulations and other items of equipment which the Administrator finds may be inoperative and yet maintain an acceptable level of safety by appropriate conditions and limitations;  it does not contain obviously required items such as wings, flaps, and rudders.  The MMEL is the basis for development of individual operator MELs which take into consideration the operator's particular aircraft equipment configuration and operational conditions. Operator MELs, for administrative control, may include items not contained in the MMEL; however, relief for administrative control items must be approved by the Administrator.  An operator's MEL may differ in format from the MMEL, but cannot be less restrictive than the MMEL.  The individual operator's MEL, when approved and authorized, permits operation of the aircraft with inoperative equipment.

Equipment not required by the operation being conducted and equipment in excess of FAR requirements are included in the MEL with appropriate conditions and limitations.  The MEL must not deviate from the Aircraft Flight Manual Limitations, Emergency Procedures or with Airworthiness Directives.  It is important to remember that all equipment related to the airworthiness and the operating regulations of the aircraft not listed on the MMEL must be operative.

Scott Dann	Page:	XVI

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Preamble

Suitable conditions and limitations in the form of placards, maintenance procedures, crew operating procedures and other restrictions as necessary are specified in the MEL to ensure that an acceptable level of safety is maintained.

The MEL is intended to permit operation with inoperative items of equipment for a period of time until repairs can be accomplished.  It is important that repairs be accomplished at the earliest opportunity.  In order to maintain an acceptable level of safety and reliability the MMEL establishes limitations on the duration of and conditions for operation with inoperative equipment.  The MEL provides for release of the aircraft for flight with inoperative equipment.  When an item of equipment is discovered to be inoperative, it is reported by making an entry in the Aircraft Maintenance Record/Logbook as prescribed by FAR.  The item is then either repaired or may be deferred per the MEL or other approved means acceptable to the administrator prior to further operation.  MEL conditions and limitations, do not relieve the operator from determining that the aircraft is in condition for safe operation with items of equipment inoperative.

When these requirements are met, an Airworthiness Release, Aircraft Maintenance Record/Logbook entry, or other approved documentation is issued as prescribed by FAR.  Such documentation is required prior to operation with any item of equipment inoperative.

Operators are responsible for exercising the necessary operational control to ensure that an acceptable level of safety is maintained.  When operating with multiple inoperative items, the interrelationships between those items and the effect on aircraft operation and crew workload will be considered.

Operators are to establish a controlled and sound repair program including the parts, personnel, facilities, procedures, and schedules to ensure timely repair.

WHEN USING THE MEL, COMPLIANCE WITH THE STATED INTENT OF THE PREAMBLE, DEFINITIONS, AND THE CONDITIONS AND LIMITATIONS SPECIFIED IN THE MEL IS REQUIRED.

Scott Dann	Page:	XVII

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Guidelines for (O) & (M) Procedures

The FOEB has identified a need for certain procedures to provide an adequate level of safety while providing relief for the following items. These procedures must be established by the operator and may be based on the aircraft manufacturer’s recommended procedures, Supplemental Type Certificate modifier’s recommended procedures, or equivalent operator procedures.  When recommended procedures are published, the operator should comply with these procedures.  If recommended procedures are not published, the following guidelines delineate the aspects to be considered by the operator in the development of required procedures.

22-1 (M) 	Procedure to assure that no electrical or mechanical fault exists that will have an adverse effect on any flight control function.

22-2 (O) 	Procedure to establish effective means to disconnect the Autopilot with inoperative Yoke Autopilot Disconnect.

22-2 (M) 	Procedure to deactivate and secure the Electric Pitch Trim System.

23-2 (O) 	Procedure to verify the Headset is operative on the audio system with the operative Speaker.

23-3 (O) 	Procedure to establish effective means to use the right side audio system for communications when the failed audio system is on the left side.

23-6 (O) 	Procedure to ensure SATCOM Voice or Data Link operates normally, alternate procedures are established and used, SATCOM coverage is available over the intended route of flight, and if INMARSAT Codes are not available while using SATCOM Voice prior coordination with the appropriate ATC facility is required.

23-7 (O) 		Procedure to establish alternate means to brief passengers when PA is inoperative.

25-2-1 (M) 		Procedure to ensure Seat is secured in the full upright position.

25-4-2-a (M) 	Procedure to disconnect the Remote ELT Switch and ensure the ELT is ARMED for operation.

25-8 (M) 		Procedure to ensure procedures are established to secure compartment closed and placard.

25-8 (M) 		Procedure to ensure door(s) is properly removed and compartment is appropriately placarded.

25-8 (O) 	Procedure to ensure Bin or Compartment is not used for storage and crew and passengers are alerted and briefed.

25-8-1 (M) 	Procedure to ensure Door Latch remains operative with inoperative Key Lock or secure door by alternate means

25-9 (M) 		Procedure to ensure acceptable cargo loading limits from an approved source are observed.

25-10 (M) 		Procedure to ensure container is empty and access to the compartment is secured.

Scott Dann	Page:	XVIII

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Guidelines for (O) & (M) Procedures

25-10 (O) 	Procedure to ensure sufficient waste receptacles are available to accommodate all waste that may be generated on the flight.

25-11 (M) 		Procedure to deactivate and secure affected EMS equipment.

27-1 (M) 	Procedure to ensure failure of the Electric Trim will not interfere with the proper operation of the Manual Trim and to isolate electrical power from the Electric Trim System.

28-1 (M) 	Procedure to check for mechanical or electrical faults which might adversely affect other fuel system components and verify the Manual Outer Tank Transfer System is operative.

Procedure to check for mechanical or electrical faults which might adversely affect other fuel system components and a procedure to render the Outer Wing Tank Transfer pumps inoperative.

28-2 (M) 	Procedure to check for mechanical or electrical faults which might adversely affect other fuel system components.

31-2 (O) 		Procedure to record hours of operation.

32-1 (O) 		Procedure to prevent movement of aircraft when stopped or parked.

33-8 (O) 		Procedure to appropriately brief passengers.

33-9 (O) 	Procedure to identify minimum sufficient operative Lighting for the crew to perform required duties and for passengers to locate items and move safely about the cabin during night operations.

34-8 (M) 		Procedure to deactivate and secure the Radar Altimeter System.

34-8 (O) 	Alternate procedures for terrain clearance awareness and approach minimums with the Radar Altimeter inoperative.

34-15(O) 		Procedure to ensure Autopilot with Altitude Hold is operative.

34-19 (M) 		Procedure to deactivate and secure the TCAS System.

34-20 (M) 		Procedure to deactivate and secure the TCAS System.

Scott Dann	Page:	XIX

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Guidelines for (O) & (M) Procedures

34-20-2 (O) 	Operations procedure to ensure TA Visual Display and Audio Functions are operative, TA ONLY mode is selected by the crew, and enroute or approach procedures do not require use of the RA Display System.

34-20-3 (O)	Operations procedure to ensure all RA Visual Display and Audio Functions are operative, and enroute or approach procedures do not require use of the TA Display System.

34-21-A-1 (O)	Operations procedure to ensure alternate procedures are established and used for inoperative GPWS.

34-21-A-1-a (O)	Operations procedure to ensure alternate procedures are established and used for inoperative modes.

34-21-A-1-d (O)	Operations procedure to ensure alternate procedures are established and used for inoperative advisory callouts.

34-21-A-1-e (O)	Operations procedure to ensure alternate procedures are established and used for inoperative windshear mode.

34-21-A-2 (O)	Operations procedure to ensure alternate procedures are established and used for inoperative

FLTA/PDA mode.

34-21-B-1 (O)	Operations procedure to ensure alternate procedures are established and used for inoperative GPWS.

34-21-B-1-a (O)	Operations procedure to ensure alternate procedures are established and used for inoperative modes.

34-21-B-1-d (O)	Operations procedure to ensure alternate procedures are established and used for inoperative advisory callouts.

34-21-B-1-e (O)	Operations procedure to ensure alternate procedures are established and used for inoperative windshear mode.

Scott Dann	Page:	XX

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Guidelines for (O) & (M) Procedures

34-21-C-1 (O)	Operations procedure to ensure pilot planning and awareness of terrain clearance.

34-22 (O)		Procedure to ensure 3 Stabilized Compass Systems are operative.

34-22 (O)		Procedure to ensure 2 Stabilized Compass Systems are operative, airplane is configured for Dual 			Independent Navigation Capability, and area of operation is under Positive Radar Control by ATC.

34-22 (O)	Procedure to determine the flight is entirely within areas of magnetic unreliability and 2 Stabilized Directional Gyro Systems are installed and operative.

34-23 (O)	Procedure to ensure that alternate procedures are established and used for inoperative windshear mode.  Alternate procedures should include reviewing windshear avoidance and windshear recovery procedures.

34-24 (O)	Procedure to ensure that alternate procedures are established and used for inoperative windshear mode.  Alternate procedures should include reviewing windshear avoidance and windshear recovery procedures.

34-25-1 (O)	Operations procedure to ensure current Aeronautical Charts are used, status and suitability of Navigation Facilities to be used are verified, and Approach Navigation Radios are manually tuned and identified.

Scott Dann	Page:	XXI

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

Guidelines for (O) & (M) Procedures

34-25-1 (O)	Operations procedure to ensure current Aeronautical Charts are used, status and suitability of Navigation Facilities to be used are verified, and Approach Navigation Radios are manually tuned and identified.

34-26-1 (O)	Operations procedure to ensure current Aeronautical Charts are used, status and suitability of Navigation Facilities to be used are verified, and Approach Navigation Radios are manually tuned and identified.

46-1 (O)	Procedure for alternate means to provide flight operating information in current and appropriate form accessible for each pilot at the pilot station.

46-1-1 (O)	Procedure for alternate means to operate the EFB without automatic data input.

46-1-2 (O)	Procedure to ensure adequate backup or Battery Power Supply is available to operate the EFB for the entire flight duration or alternate means to provide flight operating information in current and appropriate form accessible for each flight at the pilot station.

46-1-3 (M)	Procedure to ensure associated EFB and hardware is secured in an acceptable location by alternate means or removed from aircraft.

46-1-3 (O)	Procedure for alternate means to provide flight operating information in current and appropriate form accessible for each flight at the pilot station if the normal use of the EFB is not available.

Scott Dann	Page:	21-1

Minimum Equipment List	Revision:	10

Mitsubishi Model MU-2B	Date:   	06/03/2003

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

21  AIR CONDITIONING

1.	Cabin Altitude		          	C     	1   	 0    	May be inoperative for unpressurized flight

Warning System

C          1	 0 	May be inoperative for pressurized flight at or below					 				10000 feet MSL

2.	Automatic		          	C     	1   	 0    	May be inoperative, provided manual pressurization

Pressurization               				             control is operative.

Controller

C     	1   	 0    	May be inoperative for unpressurized flight

3.	Manual			          	C     	1   	 0    	May be inoperative for unpressurized flight

Pressurization

Controller

4.	Automatic			C     	1   	 0    	May be inoperative, provided manual temperature

Temperature               				             control system is operative

Control System

5.	Air Conditioning		          	C     	1   	 0    	May be inoperative for unpressurized flight provided

System                				             bleed selector is in RAM position

Scott Dann	Page:	22-1

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

22  AUTO FLIGHT

1.  	Autopilot System     		C  	1   	0   	(M) May be inoperative, provided:

a) Operations do not require its use, and

a) Aircraft is operated Day VMC.

NOTE: RVSM is not authorized.

1)	Navigation Mode		C	1	0	NOTE: Any Course Tracking Mode.

(CAPTURE/TRACK on M-4C)

2)	Approach Mode			C	1	0	NOTE: Any approach Tracking Mode.

(VOR/APPR on SPZ-500)

(GS Mode on M-4C)

3)	Back Course/REV		C	1	0	May be inoperative.

Course Mode

4)	Vertical Speed/			C	2	0	May be inoperative

Indicated Speed Function

2.  	Autopilot Disconnect/		C  	7  	4  	(O) One may be inoperative, provided:

Trim Interrupt Switch					a) Autopilot is not used below 1500 feet AGL,

(Yoke Button)						b) Approach procedures do not require the use of the									Autopilot, and

c) Trim Interruption function is verified operative at the flying

pilot station prior to each departure.

Scott Dann	Page:	22-2

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

22  AUTO FLIGHT

2.  	Autopilot Disconnect/		B  	7  	0  	(O) One may be inoperative, provided:

Trim Interrupt Switch					a) Autopilot is not used below 1500 feet AGL,

(Yoke Button)						b) Approach procedures do not require the use of the									Autopilot, and

c) Trim Interrupt function is verified operative at the flying

pilot station prior to each departure, and

d) Alternate procedures are established and used to									disconnect Autopilot/Yaw Damper.

B  	7  	0  	(M) One may be inoperative, provided:

a) Autopilot is not used,

b) Yaw Damper is not used

c) Electric Trim is deactivated, and

d) Aircraft is operated Day VMC.

3.	Yaw Damper			C  	1  	0  	May be inoperative, provided:

b) Yaw Damper is not used

a) Autopilot is not used,

c) Aircraft is operated Day VMC.

4.	Flight Director

1)	Sperry SPZ-500			--	0	0	Not Installed.

2)	Bendix M-4D			C	1	0	May be inoperative.

3)	Bendix M-4C			--	0	0	Not Installed.

Scott Dann	Page:	23-1

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

23  COMMUNICATIONS

1.  	Communications     		D  	2   	1   	Any in excess of those required by FAR 91.205 and 91.511

Equipment						may be inoperative provided it is not powered by the

(VHF and UHF)						Emergency Power Supply and not required for emergency

procedures.

2.  	Cockpit Speakers    		C	2	1	(O) One may be inoperative provided Headset with Boom 								Microphone remains operative on affected audio system.

C  	2   	0   	May be inoperative, provided:

a) Procedures do not require their use, and

b) Headsets are installed and operate normally.

3.  	Audio Panel/Amplifier    		C	2	1	(O) One may be inoperative provided aircraft is operated VFR

C  	2   	0   	May be inoperative, provided:

a) Procedures do not require their use, and

b) Headsets are installed and operate normally.

4.  	Cockpit Voice    		--  	0   	0   	Not installed

Recorder (CVR)

System

5.  	Boom Microphones   		--  	0   	0   	Not installed

Scott Dann	Page:	23-2

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

23  COMMUNICATIONS

6.  	High Frequency (HF)     		-- 	0   	0   	Not installed

Communication

System

Scott Dann	Page:	23-3

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

23  COMMUNICATIONS

7.  	Passenger Address System	--  	0   	0   	Not utilized

(PA)

Scott Dann	Page:	23-4

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

23  COMMUNICATIONS

8.  	Hand Held Microphones		C  	2   	1   	Right side may be inoperative provided Second-in-Command

is not required.

C	2	1	One may be inoperative provided a Boom Microphone is

Available and operative at the affected flight deck position.

9.  	Static Discharge Wicks		C  	10   	9   	One may be damaged or missing provided the aircraft is

operated VMC.

Scott Dann	Page:	24-1

Minimum Equipment List	Revision:	9

Mitsubishi Model MU-2B	Date:   	03/25/1994

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

24  ELECTRICAL POWER

1.  	Inverter Failure     		-  	-   	-   	DELETED, Revision 8

Warning Light

2.  	Generator Warning     		-  	-   	-   	DELETED, Revision 8

Lights

Scott Dann	Page:	25-1

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

25  EQUIPMENT/FURNISHINGS

1.  	Cockpit Shoulder     		B  	2   	1   	Right side may be inoperative, provided seat remains

Harness						unoccupied

2.  	Passenger Seat			D 	5	0   	May be inoperative, provided:

a) Seat does not block an Emergency Exit, and

b) Seat does not restrict any passenger from access to the

main aircraft aisle, and

c) Affected seat(s) are blocked and placarded

“Do Not Occupy".

NOTE: 	A seat with inoperative seat belt or shoulder harness 									is considered to be inoperative

1) Recline Mechanism   			--  	0  	0   	Not installed.

2) Armrest	   			D  	3  	0   	May be inoperative or missing and seat occupied provided:

a) Armrest does not block an Emergency Exit,

b) Armrest does not restrict any passenger from access to the

main aircraft aisle, and

c) For an armrest with a Recline Mechanism, seat is secured

in the upright position.

3.  	Flotation/Overwater Eqpt

Flotation Devices (TSO-C72)		D  	6  	0   	May be inoperative, provided enroute over water operations

do not require their use, as per FAR 91.509.

Life Rafts				D	1	0	May be inoperative, provided enroute over water operations

do not require their use, as per FAR 91.509.

Scott Dann	Page:	25-2

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

25  EQUIPMENT/FURNISHINGS

4.  	Emergency Locator

Transmitter (ELT)

1)	Survival Type ELT’s		--	0	0	Not Installed.

2)	Fixed ELT’s			A	1	0	May be inoperative or missing provided repairs are made

within ninety days.

a)	Remote Switch			D	1	0	(M) May be inoperative provided:

a) Remote switch is disconnected from the ELT, and

b) ELT switch is placed in the ARM position

5.  	Emergency Medical   		--  	0   	0   	Not installed

Equipment

Scott Dann	Page:	25-3

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

25  EQUIPMENT/FURNISHINGS

6.  	Non-Essential Equipment	--	--	0	May be inoperative, damaged or missing provided that the

And Furnishings (NEF)					item(s) are deferred in accordance with the operator’s NEF

deferral program.  The NEF program, procedures, and

processes are outlined in the operator’s manual.  (M) and (O)

procedures, if required, must be available to the flight crew

and included in the operator’s appropriate document.

NOTE: Exterior Lavatory Door Ash Trays are not considered

NEF items.

7.	“Fasten Seat Belt		C	2	1	One or more signs or placards may be illegible or missing

While Seated” sign 					provided a legible sign or placard is visible from each

or Placard						occupied passenger seat.

Scott Dann	Page:	25-4

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

25  EQUIPMENT/FURNISHINGS

8.	Cabin Storage			C	2	0	(M) May be inoperative, provided:

Compartment/Bins/					a) Procedures are established to secure the

Closets							compartment closed,

b) Associated compartment is placarded “DO NOT USE”,

c) Any emergency equipment located in affected

compartment is considered inoperative, and

d) Affected compartment is not used for storage of any

item(s) except for those permanently affixed.

(M)(O) May be inoperative provided:

a) Affected door is removed,

b) Associated compartment is not used for storage of any

items, except those permanently affixed,

c) Associated compartment is placarded “DO NOT USE”,

d) Passengers are briefed that associated compartment is

not used.

NOTE: Any permanently affixed Emergency Equipment

located in the associated storage compartment is available

for use.

1) Storage Comp’t. Key Locks		--	0	0	Not installed

9.	Cargo Restraint Systems	--	0	0	Not installed

10.	Galley Waste			--	0	0	Not installed

Receptacles/Access Doors/

Covers

Scott Dann	Page:	25-5

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

25  EQUIPMENT/FURNISHINGS

11.	Emergency Medical Services	--	0	0	Not installed

(EMS) Equipment

Scott Dann	Page:	26-1

Minimum Equipment List	Revision:	10

Mitsubishi Model MU-2B	Date:   	06/03/2003

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

26  FIRE PROTECTION

1.  	Portable Fire Extinguisher	D	1	1	Must be operative.

Scott Dann	Page:	27-1

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

27  FLIGHT CONTROLS

1.  	Electric Pitch Trim System	C	1	0	(M) May be inoperative provided:

a) Manual Pitch Trim is verified operative, and

b) Autopilot is considered inoperative

1)	Electric Pitch Trim Switches	C	2	0	May be inoperative provided Manual Pitch Trim is verified

(Yoke)							operative.

Scott Dann	Page:	28-1

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

28  FUEL

1.  	Automatic Fuel			C	2	0	(M) May be inoperative, provided:

Transfer Systems					a) Tip tanks are filled to a maximum of 400 lbs/60 US gallons 	(Installed only on					each,

A10SW Aircraft)						b) Both manual tip tank transfer systems are operative.

c) Outer Wing Tank quantity does not exceed 150 pounds in

each Outer Tank, and

d) Manual Outer Tank Transfer System is installed and

operative.

C	2	0	(M) May be inoperative, provided:

a) Tip tanks are filled to a maximum of 400 lbs/60 US gallons 								each,

b) Both manual tip tank transfer systems are operative.

c) Outer Wing Tank quantity does not exceed 150 pounds in

each Outer Tank, and

d) Outer Tank Fuel is considered unusable.

2.	Tip Tanks Manual		C	2	0	(M) May be inoperative, provided tip tanks are empty at

Transfer Systems					takeoff.

Scott Dann	Page:	30-1

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

30  ICE AND RAIN PROTECTION

1.  	Propeller Deicing		C	2	0	May be inoperative, provided aircraft is not operated in known

Systems						or forecast icing conditions.

2.  	Stall Warning Heater		C	1	0	May be inoperative, provided aircraft is not operated in known

or forecast icing conditions.

3.  	Static Port Heaters		C	2	0	May be inoperative, provided aircraft is not operated in known

(US type certificated					or forecast icing conditions.

aircraft only)

4.  	Loadmeter and Selector for	C	1	0	May be inoperative, provided aircraft is not operated in known

Propeller Heat, Pitot Heat,				or forecast icing conditions.

Static Port Anti-Ice System,

And Stall Warning Anti-Ice

System (US type certificated

aircraft only)

5.	Windshield Ice Protection	C	1	0	May be inoperative, provided aircraft is not operated in known

System (Anti-Ice Fluid or				or forecast icing conditions.

Heat System)

Scott Dann	Page:	30-2

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

30  ICE AND RAIN PROTECTION

6.	Surface Deicing System		C	1	0	May be inoperative, provided manual system is operative.

(Automatic)

(Wing, Vertical, and		C	1	0	May be inoperative, provided aircraft is not operated in known

Horizontal Stabilizer)					or forecast icing conditions.

7.	Surface Deicing System		C	1	0	May be inoperative, provided aircraft is not operated in known	(Manual)						or forecast icing conditions.

(Wing, Vertical, and

Horizontal Stabilizer)

8.  	Engine Oil Cooler		C	2	0	May be inoperative, provided aircraft is not operated in known

Anti-Ice Systems					or forecast icing conditions.

9.	Windshield Wipers		C	2	0	May be inoperative, provided flight is not conducted in

precipitation within 5 nm of the airport of takeoff or

intended landing.

10.	Pitot Heaters			C	2	1	Right side may be inoperative provided:

a) Second-in-Command is not required, and

b) Aircraft is operated VMC.

B	2	0	May be inoperative provided:

a) Aircraft is operated VFR only, and

b) Aircraft is not operated in known or forecast icing

conditions.

11.	Ice Detector Systems		--	0	0	Not installed

Scott Dann	Page:	31-1

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

31  INDICATING/RECORDING

SYSTEMS

1.  	Clock with sweep second	C	1	0	May be inoperative for VFR operations.

Hand or electric digital clock.

2.	Flight Hour Recorder		C	1	0	(O) May be inoperative, provided a procedure is established

for the crew to record the time in service for each leg.

3.	Flight Data Recorder (FDR)	--	0	0	Not installed

Scott Dann	Page:	31-2

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

31  INDICATING/RECORDING

SYSTEMS

Not Used				--	--	--

Scott Dann	Page:	32-1

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

32  LANDING GEAR

1.	Parking Brake			C	1	0	(O) May be inoperative.

Scott Dann	Page:	33-1

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

33  LIGHTS

1.  	Position Lights			C	3	0	May be inoperative for day operations.

1)	Tail Position Light (White)	C	1	0	May be inoperative provided the Anti-Collision Light (Strobe)

is ON and operative for night operations.

2.	Anti-Collision Beacon		B	1	0	May be inoperative for day operations.

Light System

3.  	Landing Lights			C	2	1	May be inoperative.

C	2	0	May be inoperative for day operations.

1)	Landing Light Extension		C	2	0	May be inoperative provided AFM Limits are observed if in

Systems						other than retracted position.

C	2	0	May be inoperative provided Landing Light is considered

inoperative if in other than extended position.

4.	Cockpit/Flight Deck		C	8	5	Individual lights may be inoperative provided remaining

Flight Compartment					lights are:

and Instrument Lighting					a) Sufficient to clearly illuminate all required instruments,

System							    controls, and other devices for which it is provided

b) Positioned so that direct rays are shielded from flight

crew members eyes, and

c) Lighting configuration and intensity is acceptable to the

flight crew.

5.	Wing Ice Lights			C	1	0	May be inoperative, provided:

a) Aircraft is not operated in known or forecast icing

conditions at, and

b) Ground deicing procedures do not require the use of Wing 								Ice Lights.  (AD96-25-02)

6.	Taxi Lights			C	2	0	May be inoperative, provided aircraft is not operated at

night in known or forecast icing conditions.  (AD96-25-02)

7.	Strobe Light System		C	1	0	May be inoperative.

Scott Dann	Page:	33-2

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

33  LIGHTS

8.	Passenger Notice System	C	1	0	(O) May be inoperative, provided appropriate verbal

(No Smoking -						      briefings are given to passengers.

Fasten Seat Belt)

9.	Cabin Interior Lights		C	4	0	(O) May be partially inoperative provided,

a) Cabin Emergency Lighting is operative.

b) Sufficient lighting is available for crew to perform required 								duties, and

c) Sufficient lighting is operative for passenger-carrying

operations at night.

10.	Baggage Compartment		D	2	0	May be inoperative.

Light System

Scott Dann	Page:	34-1

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

34  NAVIGATION

1.  	Altimeter Barometric		B	2	1	May be inoperative on the right side, provided:

Pressure Adjustable					a) Second in Command is not required, and

b) Aircraft is not equipped with EFIS, Air Data Display Unit(s)

or Servoed Electric Altimeter(s).

2.	Airspeed Indicator		C	2	1	May be inoperative on the right side, provided:

Pressure Adjustable					a) Second in Command is not required, and

b) Aircraft is not equipped with EFIS

3.	Gyroscopic Pitch and		B	2	1	May be inoperative on the right side, provided:

Bank Indicator System					a) Second in Command is not required, and

b) Aircraft is not equipped with EFIS

4.	Gyroscopic Directional		B	2	1	May be inoperative on the right side, provided:

Indicator System					a) Second in Command is not required, and

b) Aircraft is not equipped with EFIS

5.	Gyroscopic Rate of		B	2	1	May be inoperative on the right side, provided:

Turn/Slip Skid Indicator					a) Second in Command is not required, and

(M4D)							b) Autopilot is considered inoperative

B	2	0	May be inoperative, provided:

a) Aircraft is operated Day VFR, and

b) Autopilot is considered inoperative

6.	Vertical Speed Indicator		B	2	1	May be inoperative on the right side, provided Second in

Command is not required

B	2	0	May be inoperative, provided aircraft is operated Day VFR

7.	Flight Director						DELETED Revision 11, See Chapter 22

8.	Radar Altimeter			C	1	0	(M)(O) May be inoperative.

9.	Weather Radar/			C	1	0	May be inoperative.

Thunderstorm Detection

Equipment

Scott Dann	Page:	34-2

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

34  NAVIGATION

10.	ATC Transponder and		C	2	0	May be inoperative, provided:

Automatic Altitude					a) Enroute operations do not require its use, and

Reporting System					b) Prior to flight, approval is obtained from ATC

facilities having jurisdiction over the planned

route of flight.

NOTE: RVSM is not authorized

D	2	1	Any in excess of those required by FAR may be inoperative.

1)	Elementary and Enhanced	--	0	0	Not installed.

Downlink Aircraft Reportable

Parameters not required by

FAR

2)	ADS-B Squitter			--	0	0	Not installed.

Transmissions

3)	Control Wheel Transponder	C	2	0	May be inoperative provided Transponder Ident Switch is

Ident Switch						operative.

11.	Marker Beacon			C	1	0	May be inoperative, provided approach procedure does not

Require its use.

12.	Altitude Encoder					Included with Item 10.

13.	Navigation Equipment

a)	VOR/ILS		C	2	0	May be inoperative, provided enroute navigation and									approach procedures do not require its use.

b)	GPS			C	2	0	May be inoperative, provided enroute navigation and									approach procedures do not require its use.

14.	Distance Measuring		D	1	0	Any in excess of those required for enroute navigation and

Equipment (DME) Systems				approach procedures may be inoperative.

15.	Altitude Alerting System		A	1	0	(O) May be inoperative, provided:

a) Autopilot with altitude hold is operative,

b) Enroute operations do not require its use, and

c) Repairs are made within three flight days.

C	1	0	May be inoperative.

Scott Dann	Page:	34-3

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

34  NAVIGATION

16.	ADF				C	1	0	May be inoperative, provided enroute navigation and									approach procedures do not require its use.

17.	RMI				C	1	0	May be inoperative, provided enroute navigation and									approach procedures do not require its use.

18.	Traffic Collision Avoidance	--	0	0	Not installed

Detection (TCAD)

19.	Traffic Alert and Collision	--	0	0	Not installed

Avoidance System (TCAS I)

Scott Dann	Page:	34-4

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

34  NAVIGATION

20.	Traffic Alert and Collision	--	0	0	Not installed

Avoidance System (TCAS II)

Scott Dann	Page:	34-5

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 3935A

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

34  NAVIGATION

21.	Terrain Awareness and		--	0	0	Not installed

Warning System (TAWS)

Ground Proximity and

Warning System (TAWS)

Scott Dann	Page:	34-6

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

34  NAVIGATION

22.	Nonstabilized Magnetic		B	1	0	(O) May be inoperative provided any combination of three o

Compass						or IRU/AHRS stabilized compass systems are operative.

B	1	0	(O) May be inoperative, provided:

a) Any combination of two gyro or INS/AHRS stabilized

compass systems are operative, and

b) Aircraft is operated with dual independent navigation

capability and under positive radar control by ATC on

the enroute portion of the flight.

B	1	0	(O) May be inoperative for flights that are entirely within areas

of magnetic unreliability provided at least two stabilized

directional gyro systems are installed, operative and used

in conjunction with approved free gyro navigation techniques.

Scott Dann	Page:	34-7

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

34  NAVIGATION

23.	Windshear Warning and		--	0	0	Not installed

Flight Guidance System

24.	Windshear Detection and	--	0	0	Not installed

Avoidance System

Scott Dann	Page:	34-8

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

34  NAVIGATION

25.	Flight Management		--	0	0	Not installed

System Navigation

Databases

26.	Navigation Management	C	1	0	(O) May be out of currency, provided:

System							a) Current aeronautical charts are used to verify

1) Navigation Databases				navigation fixes prior to dispatch,

b) Procedures are established and used to verify

status and suitability of navigation facilities used

to define the route of flight, and

c) Approach navigation radios are manually tuned

and identified.

Scott Dann	Page:	34-9

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

34  NAVIGATION

27.	Automatic Dependent		--	0	0	Not installed

Surveillance Broadcast

System (ADS-B)

Scott Dann	Page:	34-10

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

34  NAVIGATION

28.	Standby Attitude Indicator	C	1	0	May be inoperative.

B	1	0	May be inoperative, provided:

a) Operations are conducted in Day VMC only, and

b) Operations are not conducted into known or forecast

VFR-on-Top conditions.

Scott Dann	Page:	34-11

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

34  NAVIGATION

Not Used				--	--	--

Scott Dann	Page:	34-12

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

34  NAVIGATION

Not Used				--	--	--

Scott Dann	Page:	34-13

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

34  NAVIGATION

Not Used				--	--	--

Scott Dann	Page:	34-14

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

34  NAVIGATION

Not Used				--	--	--

Scott Dann	Page:	34-15

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

34  NAVIGATION

Not Used				--	--	--

Scott Dann	Page:	35-1

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

35  OXYGEN

1.	Oxygen System						DELETED Rev. 11

1)	Passenger Oxygen Masks	C	6	0	May be inoperative provided:

a) Corresponding seat is blocked and placarded “DO NOT

OCCUPY”, and

b) Affected mask is not connected to oxygen system.

NOTE: Cockpit crew oxygen masks must be operative for

Each required pilot.

2.	Protective Breathing		--	0	0	Not installed

Equipment (PBE)

Scott Dann	Page:	46-1

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

46  INFORMATION SYSTEMS

1.	Electronic Flight Bags (EFB)	--	0	0	Not installed

Scott Dann	Page:	52-1

Minimum Equipment List	Revision:	11

Mitsubishi Model MU-2B	Date:   	02/06/2009

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

52  DOORS

1.	Entrance Door Open		C	1	0	May be inoperative, provided:

Warning Light						a) A flight crewmember confirms, by visual inspection, that

(Red Light or ENT DOOR				    all doors are latched and secure, prior to each takeoff

OPEN Annunciator)

b) The fasten seat belt sign remains ON or, in the event

the Fasten Seat Belt sign is inoperative, a verbal briefing

to remain seated in given,

Scott Dann	Page:	61-1

Minimum Equipment List	Revision:	9

Mitsubishi Model MU-2B	Date:   	03/25/1994

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

61  PROPELLERS

1.	Propeller Synchrophaser	C	1	0	May be inoperative.

Scott Dann	Page:	76-1

Minimum Equipment List	Revision:	9

Mitsubishi Model MU-2B	Date:   	03/25/1994

N82WC; s/n- 393SA

1. CATEGORY

2. NUMBER INSTALLED

SYSTEM &

SEQUENCE NUMBERS                  			3. NUMBER REQUIRED FOR DISPATCH

4. REMARKS OR EXCEPTIONS

76  ENGINE CONTROLS

1.	Torque and Temperature	C	2	0	May be inoperative provided AFM limits are not exceeded.

Limiter